# SOP for Rigol DG1022Z Stimulation GUI

## Purpose

This software controls the Rigol DG1022Z Function Generator to deliver repeated sine burst stimulation with synchronized TTL output from the SYNC connector.

## Equipment

- Laptop with Stimulation GUI

- Rigol DG1022Z Function Generator

- USB data cable

- BNC cables

- Experimental setup

## Preparation

- Turn ON the Rigol DG1022Z.

- Connect the generator to the laptop using the USB cable.

- Close UltraSigma or any Rigol software.

- Open the Stimulation GUI.

## Connection

- Leave the VISA Address field unchanged (or blank).

- Press CONNECT.

- Status should display Connected.

Note: Generator front panel may lock during remote control — this is normal.

## Set Parameters

Enter the stimulation values:

- Freq (Hz): sine frequency

- Duration (s): burst length

- Amplitude (Vpp): output voltage

- Interval (s): pause between bursts

- Reps: number of repetitions

## Run Stimulation

- Press RUN.

- Progress bar shows stimulation status:

  - Blue = Running

  - Green = Finished

- SYNC OUT sends TTL during stimulation.

## Stop Stimulation

Press STOP at any time to immediately turn OFF output.

## Disconnect

After finishing:

- Press DISC.

- Generator returns to manual control.

## UltraSigma (Optional)

UltraSigma can be used separately to:

- Manually test settings

- View SCPI commands

⚠ Do not run UltraSigma at the same time as the GUI.

## Safety

- Verify amplitude before connecting to experimental hardware.

- Do not connect or remove cables during active output.

- Ensure proper grounding.

## Troubleshooting

Cannot connect

- Check USB cable

- Restart generator

- Close UltraSigma

No signal

- Confirm RUN was pressed

- Check amplitude and cables