# Quick Run Guide for Rigol DG1022Z Stimulation GUI

- Setup

- Install python 3.xx

- Install Ultrasigma driver from Rigol website

- Go to https://github.com/HamedShabani/Rigol_FG_stim and download the file rigol_fg_gui_final2.py

✔ Turn ON the Rigol DG1022Z
✔ Connect USB cable to laptop
✔ Connect BNC cables to setup
✔ Close UltraSigma before using the GUI

Open the Stimulation GUI.

## 2. Connect

- Leave VISA Address unchanged (or blank).

- Press CONNECT.

Status should show Connected.

(Generator buttons may lock  this is normal.)

## 3. Enter Stimulation Parameters

| Field | Meaning |
| --- | --- |
| Freq (Hz) | Sine frequency |
| Duration (s) | Length of each burst |
| Amplitude (Vpp) | Output voltage |
| Interval (s) | Pause between bursts |
| Reps | Number of repetitions |