# Magnetic Flux Calibration Tool – Operator Quick Guide (SOP)

## Purpose

This tool calculates the Function Generator Voltage (Vpp) required to achieve a desired magnetic field B (mT) using pre-measured calibration data.

⚠️ The voltage shown is FG voltage BEFORE the amplifier.
⚠️ The amplifier gain is fixed and must NOT be changed.

## Data Source

Interpolation values are taken from the Excel file:

“B_flux_Table”
based on measurements of the large coil.

## ⚠️ Safety Attention

❗ Do NOT enter or apply more than 5 Vpp from the Function Generator.
Higher voltage may damage the amplifier.

## How to Use

- Open the program (EXE or HTML file).

- Select the Frequency (Hz):

  - 5 Hz

  - 10 Hz

- Select the Mode:

  - B → FG Voltage → Find required FG voltage for a target field.

  - FG Voltage → B → Estimate magnetic field from a known FG voltage.

- Enter the value:

  - Target B (mT) or

  - FG Voltage (Vpp)

- Press Compute.

The tool will display:

- Required FG Voltage (Vpp)

- Expected Coil Current (App)

- Estimated Magnetic Flux (mT)

## Important Notes

- Operator should ONLY adjust FG Voltage on the generator.

- Do NOT change amplifier gain.

- Do NOT exceed 5 Vpp.

- Results are based on interpolation of calibration data from B_flux_Table.

- Values outside measured range require Allow Extrapolation.

## Example

Target field = 5 mT at 10 Hz
→ Select 10 Hz
→ Mode = B → FG Voltage
→ Enter 5
→ Press Compute

## If Something Looks Wrong

- Check correct frequency is selected.

- Verify FG output is enabled.

- Confirm calibration file/version.

- Ask Hamed if unsure.